- Section 102113 
Toilet Compartments - Hadrian

PART 1  GENERAL

Section Includes

Metal toilet compartments.

Phenolic toilet compartments.

Solid plastic toilet compartments.

Related Requirements

Section << 051200 - Structural Steel Framing>>.

Section << 061000 - Rough Carpentry>>:  Blocking and supports.

Reference Standards

ADA Standards - 2010 ADA Standards for Accessible Design; 2010.

ASTM A653/A653M - Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process; 2025a.

ASTM A666/A666M - Standard Specification for Annealed or Cold-Worked Austenitic Stainless Steel Sheet, Strip, Plate, and Flat Bar; 2024.

ASTM B221 - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes; 2021.

ASTM B221M - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes (Metric); 2021.

ASTM B86 - Standard Specification for Zinc and Zinc-Aluminum (ZA) Alloy Foundry and Die Castings; 2023.

ASTM E84 - Standard Test Method for Surface Burning Characteristics of Building Materials; 2026.

NFPA 286 - Standard Methods of Fire Tests for Evaluating Contribution of Wall and Ceiling Interior Finish to Room Fire Growth; 2024.

Administrative Requirements

Coordination:  Coordinate work with placement of support framing and anchors in walls and ceilings.

Submittals

See Section << 013000 - Administrative Requirements>> for submittal procedures.

Product Data:  Show panel construction, hardware, and accessories.

Shop Drawings:  Indicate partition plan, elevation views, dimensions, details of ​<< ceiling;  floor;  wall; and  _____>>​ supports, and door swings.

Samples:  Submit ​<< two; or  _____>>​ samples of partition panels, ​<< 2 by 2-3/4 inches (51 by 70 mm);  2-1/2 by 4-3/4 inches (64 by 121 mm);  4 by 4 inches (102 by 102 mm); or  _____ by _____ inches (_____ by _____ mm)>>​, illustrating finish.

Manufacturer's Instructions:  Indicate << installation special procedures;  perimeter conditions requiring special attention; and  _____>>.

Specimen warranty.

Executed warranty.

Delivery, Storage, and Handling

See Section << 017419 - Construction Waste Management and Disposal>> for packaging waste requirements.

Deliver and store materials in manufacturer's unopened packaging, with labels intact.

Warranty

See Section << 017800 - Closeout Submittals>> for additional warranty requirements.

Stainless Steel Compartment Manufacturer Warranty:  Provide << 5-year; or  _____>> manufacturer warranty against material defects. Complete forms in <<Owner>>'s name and register with manufacturer.

Powder Coated Compartment Manufacturer Warranty:  Provide << 3-year; or  _____>> manufacturer warranty against material defects. Complete forms in <<Owner>>'s name and register with manufacturer.

Phenolic Compartment Manufacturer Warranty:  Provide << 25-year; or  _____>> manufacturer warranty against material defects. Complete forms in <<Owner>>'s name and register with manufacturer.

Plastic Compartment Manufacturer Warranty:  Provide << 25-year; or  _____>> manufacturer warranty against material defects. Complete forms in <<Owner>>'s name and register with manufacturer.

PART 2  PRODUCTS

Manufacturers

Hadrian<< ; _____; or  None - N/A>>:  www.hadrian-inc.com/#sle.

<< _____>>.

<< _____>>.

Substitutions:  << Section 016000 - Product Requirements; or  Not permitted>>.

Source Limitations:  Furnish products << produced by single manufacturer;  produced by same manufacturer as _____;  obtained from single supplier; and  _____>>.

Metal Toilet Compartments

Description:  Factory-fabricated doors, pilasters, and divider panels made of ​<< galvannealed, powder-coated steel;  stainless steel; or  _____>>​ faces laminated to honeycomb core; corners mitered, welded, and ground smooth; ​​​​<< floor mounted headrail braced;  pedestal mounted;  floor to ceiling;  floor mounted;  ceiling hung; or  _____>>​​​​.

Comply with <<ADA Standards>>.

Color:  ​<< As selected by Architect from manufacturer's standard range;  500 white;  504 linen;  510 black;  532 latte;  535 light grey;  545 charcoal;  541 extra white;  576 ruby red;  581 moss green;  585 sahara;  603 almond;  607 bordeaux;  609 colonial blue;  621 slate;  639 sapphire blue;  814 iron metallic;  826 kelim beige;  828 dovetail;  829 desert;  831 nickel metallic;  833 tricorn black;  837 black fox; or  _____>>​.

Finish:  ​​<< No.4 brushed finish;  5WL embossed finish;  No.4 brushed finish on inside, 5WL embossed finish on outside; or  _____>>​​.

Door and Panel Dimensions:

Steel Face Thickness:  << 22 gauge, 0.0299 inch (0.76 mm); or  _____ gauge, _____ inch (_____ mm)>>, minimum.

Core Thickness:  << 1 inch (25 mm); or  _____ inch (_____ mm)>>.

Height:  ​<< 58 inches (1.47 m);  64 inches (1.63 m);  72 inches (1.83 m);  84 inches (2.13 m);  92 inches (2.34 m); or  _____ inches (_____ m)>>​.

Door Width:  ​<< 24 inches (610 mm); or  _____ inches (_____ mm)>>​.

Door Width for Accessible Compartments:  ​<< 36 inches (914 mm); or  _____ inches (_____ mm)>>​​<< , outswinging;  , _____; or  None - N/A>>​.

Pilasters:

Steel Face Thickness:  << 22 gauge, 0.0299 inch (0.76 mm);  20 gauge, 0.0359 inch (0.91 mm);  18 gauge, 0.0478 inch (1.21 mm); or  _____ gauge, _____ inch (_____ mm)>>, minimum.

Core Thickness:  << 1-1/4 inch (32 mm); or  _____ inch (_____ mm)>>.

Reinforce pilaster tops with << 20 gauge, 0.0359 inch (0.91 mm); or  _____ gauge, _____ inch (_____ mm)>> channel.

Provide locking bars of same material as doors, with corners mitered, welded, and ground smooth.

Screens:  Same material and finish as  doors.

Mounting Type:  ​<< Wall mounted;  Floor mounted;  Pedestal mounted;  Floor to ceiling post mounted; or  _____>>​.

Height:  << 48 inches (1.22 m)>>.

Height:  ​​​<< 58 inches (1.47 m);  64 inches (1.63 m); or  _____ inches (_____ m)>>​​​.

Depth:  ​<< 18 inches (457 mm);  24 inches (610 mm); or  _____ inches (_____ mm)>>​.

Splash Plates:  Stainless steel.

Size:  << 24 by 30 inches (610 by 762 mm);  24 by 36 inches (610 by 914 mm);  36 by 30 inches (914 by 762 mm);  36 by 36 inches (914 by 914 mm); or  _____ by _____ inches (_____ by _____ mm)>>.

Provide internal reinforcement in areas with attached hardware and fittings. Mark reinforcement locations for partition-mounted washroom accessories.

Phenolic Toilet Compartments

Description:  Factory-fabricated, black core, solid phenolic doors, pilasters, and divider panels composed of melamine-impregnated surface papers pressed over phenolic-resin-impregnated core sheets and bonded under high pressure and high temperature; ​<< floor mounted headrail braced;  pedestal mounted;  floor to ceiling;  floor mounted;  ceiling hung; or  _____>>​.

Comply with <<ADA Standards>>.

Maximum Flame Spread Index: ​<< Class A, 0 to 25;  Class B, 26 to 75; or  _____>>​ when tested in accordance with <<ASTM E84>>.

Maximum Smoke Developed Index:  450 when tested in accordance with <<ASTM E84>>.

Door Color:  ​<< As selected by Architect from manufacturer's standard range;  310 ultra white;  313 daylight;  316 sheer mesh;  323 natural weft;  315 dark grey;  311 black;  312 dove grey;  314 aluminum;  317 field elm;  318 florence walnut;  319 pewter mesh;  320 portico teak;  321 graphite nebula;  324 citadel warp;  325 graphite grafix;  326 weathered ash;  327 espresso pear;  328 silveretta;  329 cello; or  ____>>​.

Pilaster Color:  ​<< As selected by Architect from manufacturer's standard range;  310 ultra white;  313 daylight;  316 sheer mesh;  323 natural weft;  315 dark grey;  311 black;  312 dove grey;  314 aluminum;  317 field elm;  318 florence walnut;  319 pewter mesh;  320 portico teak;  321 graphite nebula;  324 citadel warp;  325 graphite grafix;  326 weathered ash;  327 espresso pear;  328 silveretta;  329 cello; or  ____>>​.

Door and Panel Dimensions:

Thickness:  ​<< 3/4 inch (19 mm); or  _____ inch (_____ mm)>>​ for doors, ​<< 1/2 inch (13 mm); or  _____ inch (_____ mm)>>​ for panels.

Height:  ​<< 58 inches (1.47 m);  72 inches (1.83 m);  84 inches (2.13 m);  92 inches (2.34 m); or  _____ inches (_____ m)>>​.

Door Width:  ​<< 24 inches (610 mm); or  _____ inches (_____ mm)>>​.

Door Width for Accessible Compartments:  ​<< 36 inches (914 mm); or  _____ inches (_____ mm)>>​​​<< , outswinging;  , _____; or  None - N/A>>​​.

Pilasters:

Thickness:  ​<< 3/4 inch (19 mm); or  _____ inch (_____ mm)>>​.

Screens:  Same material and finish as  doors.

Mounting Type:  ​<< Wall mounted;  Floor mounted;  Pedestal mounted;  Floor to ceiling post mounted; or  _____>>​.

Height:  << 48 inches (1.22 m)>>.

Height:  ​<< 58 inches (1.47 m);  64 inches (1.63 m); or  _____ inches (_____ m)>>​.

Thickness:  ​<< 1 inch (25 mm); or  _____ inch (_____ mm)>>​.

Depth:  ​<< 18 inches (457 mm);  24 inches (610 mm);  46 inches (1168 mm); or  _____ inches (_____ mm)>>​.

Height:  ​<< 48 inches (1.22 m);  Match partition height;  _____ inches (_____ m); or  _____>>​.

Solid Plastic Toilet Compartments

Description:  Factory-fabricated doors, pilasters, and divider panels made of solid HDPE, ​<< floor mounted headrail braced;  floor to ceiling;  ceiling hung; or  _____>>​; comply with <<NFPA 286>>.

Comply with <<ADA Standards>>.

Maximum Flame Spread Index: ​<< Class A, 0 to 25;  Class B, 26 to 75; or  _____>>​ when tested in accordance with <<ASTM E84>>.

Maximum Smoke Developed Index:  450 when tested in accordance with <<ASTM E84>>.

Color:  ​<< As selected by Architect from manufacturer's standard range;  211 black;  212 blueberry;  213 bone;  215 burgundy;  217 canyon granite;  218 fossil;  219 frost granite;  222 hunter;  223 linen;  225 mocha;  227 paisley;  231 slate;  238 almond;  239 gray; or  _____>>​.

Door and Panel Dimensions:

Thickness:  << 1 inch (25 mm); or  _____ inch (_____ mm)>>.

Height:  ​<< 55 inches (1.39 m); or  _____ inches (_____ m)>>​.

Door Width:  ​<< 24 inches (610 mm); or  _____ inches (_____ mm)>>​.

Door Width for Accessible Compartments:  << 32 inches (813 mm);  36 inches (914 mm); or  _____ inches (_____ mm)>><< , outswinging;  , _____; or  None - N/A>>.

Pilasters:

Thickness:  << 1 inch (25 mm); or  _____ inch (_____ mm)>>.

Screens:  Same material and finish as doors.

Mounting Type:  ​<< Floor mounted;  Floor to ceiling post mounted;  Wall mounted; or  _____>>​.

Thickness:  << 1 inch (25 mm); or  _____ inch (_____ mm)>>.

Depth:  ​<< 18 inches (457 mm);  24 inches (610 mm);  46 inches (1168 mm); or  _____ inches (_____ mm)>>​.

Height:  ​<< 48 inches (1219 mm);  55 inches (1397 mm); or  _____ inches (_____ mm)>>​.

Hardware

Hardware for Stainless Steel Compartments:   Chrome-plated zinc die castings, unless specified otherwise.

Door Latches:  Concealed, with turn slot for emergency access from exterior; provide thumbturn lever for barrier-free doors.

Indicator Latches:  Manufacturer's standard ​​<< brushed nickel;  matte black; or  _____>>​​ zamac, shows red in view window when door latch in locked position, shows green when door latch in unlocked position.

Hinges:  Gravity type, adjustable, wraparound style, with threaded top hinge pin and self-lubricating nylon sleeve.

Coat Hooks:  Provide one per compartment, with rubber bumper.

Door Pulls:  Provide one for each outswinging door and each inswinging accessible compartment door.

Strikes and Keepers:  Mounted on pilaster in alignment with door latch; include << 3/4-inch (19 mm)>> diameter rubber bumper.

Hardware for Powder Coated Compartments:  Chrome-plated zinc die castings, unless specified otherwise.

Door Latches:  Concealed, with turn slot for emergency access from exterior; provide thumbturn lever for barrier-free doors.

Indicator Latches:  Manufacturer's standard ​<< brushed nickel;  matte black; or  _____>>​ zamac, shows red in view window when door latch in locked position, shows green when door latch in unlocked position.

Hinges:  Gravity type, adjustable, concealed, with threaded top hinge pin and self-lubricating nylon sleeve.

Coat Hooks:  Provide one per compartment, with rubber bumper.

Door Pulls:  Provide one for each outswinging door and each inswinging accessible compartment door.

Strikes and Keepers:  Mounted on pilaster in alignment with door latch; include << 3/4-inch (19 mm)>> diameter rubber bumper.

Hardware for Phenolic Compartments:  Stainless steel, with No.4 brushed finish; unless specified otherwise.

Indicator Latches:  Manufacturer's standard  brushed nickel zamac, shows red in view window when door latch in locked position, shows green when door latch in unlocked position.

Hinges:  Adjustable, self-closing option, surface mounted.

Coat Hooks:  Provide one per compartment; stamped style on inswing doors; flat style on outswing doors.

Door Pulls:  Provide one for each outswinging door and each inswinging accessible compartment door; manufacturer's standard brushed nickel zamac.

Wall Connections:  ​<< Continuous channels;  Stirrup brackets; or  _____>>​.

Ceiling Support Studs:  Manufacturer's standard hanging studs with << 4-inch (102 mm)>> stainless steel sleeve shoe.

Hardware for Phenolic Compartments:  Stainless steel, matte black finish; unless specified otherwise.

Indicator Latches:  Manufacturer's standard  matte black zamac, shows red in view window when door latch in locked position, shows green when door latch in unlocked position.

Hinges:  Adjustable, self-closing option, surface mounted.

Coat Hooks:  Provide one per compartment; stamped style on inswing doors; flat style with no.4 brushed finish on outswing doors.

Door Pulls:  Provide one for each outswinging door, manufacturer's standard matte black zamac.

Wall Connections:  ​<< Continuous channels;  Stirrup brackets; or  _____>>​.

Ceiling Support Studs:  Manufacturer's standard hanging studs with << 4-inch (102 mm)>> stainless steel sleeve shoe.

Hardware for Plastic Compartments:  Stainless steel, << 16 gauge, 0.0625 inch (1.59 mm)>>, unless otherwise specified.

Hinges:  Continuous with stainless steel hinge pin.

Coat Hooks:  Provide one per compartment; zinc die cast combined coat hook and bumper for inswing doors, stainless steel flat coat hook for outswing doors.

Connections:  Continuous channels.

Provide strike keeper and throw latch.

Hardware for Plastic Compartments:   Clear anodized extruded aluminum, unless otherwise specified.

Hinges:  Wrap-around style, << 1/8 inch (3.2 mm)>> thick, adjustable nylon cams on top hinges.

Coat Hooks:  Provide one per compartment; zinc die cast combined coat hook and bumper for inswing doors, stainless steel flat coat hook for outswing doors.

Connections:  Three brackets at wall connections, continuous channels at panels to pilaster connections.

Provide strike keeper and throw latch.

Pilaster Shoes:  ​<< Stainless steel;  Plastic; or  _____>>​, one-piece design, ​<< 3 inches (76 mm);  4 inches (102 mm); or  _____ inches (_____ mm)>>​ tall, concealing floor fastenings.

Pedestals:  Stainless steel.

Headrails:  Extruded aluminum, << 1 by 1-1/2 inch (25 by 38 mm); or  _____ by _____ inch (_____ by _____ mm)>> size, with  anti-grip profile and cast socket wall brackets.

Brackets:  ​<< Aluminum;  Stainless steel; or  _____>>​; ​<< manufacturer's standard type;  continuous type; or  _____>>​.

Sightline Fillers:  ​<< Aluminum;  Stainless steel; or  _____>>​, eliminates sightline gaps.

Stabilizers:  For ceiling hung partitions, provide manufacturer's standard << 3-inch (50 mm)>> stabilizer.

Fasteners:  Stainless steel<< , tamper-proof type;    _____; or  None - N/A>>.

Use through-bolts and nuts to attach panels and pilasters to brackets.

Materials

Extruded Aluminum:  <<ASTM B221>> and <<ASTM B221M>>, << 6063 alloy, T6 temper; or  _____ alloy, _____ temper>>.

Steel Sheet:  Hot-dip galvannealed steel sheet, <<ASTM A653/A653M>> Designation SS, Grade << 33 (230)>>, with << G90 (Z275); or  G_____ (Z_____)>> coating and manufacturer's standard<< anti-graffiti;    _____; or  None - N/A>> powder-coat finish.

Stainless Steel Sheet:  <<ASTM A666/A666M>>, Type 304.

Die Cast Zinc:  <<ASTM B86>>.

Accessories

Toilet Partition Suspension Members:  See Section << 051200>>.

Wood Blocking and Supports:  See Section << 061000>>.

PART 3  EXECUTION

Examination

Verify field measurements comply with manufacturer's requirements.

Verify plumbing fixture spacing and locations.

Verify locations of built-in framing, anchorage, and bracing.

Installation

Install partitions<< , screens,;  , _____ ,; or  None - N/A>> and hardware in accordance with manufacturer's instructions.

Do not perform field touch-ups of scratched or damaged finishes; replace with new materials.

Tolerances

Maximum Variation from True Position:  ​<< 1/4 inch (6 mm); or  _____ inch (_____ mm)>>​.

Maximum Variation from Plumb:  ​<< 1/8 inch (3 mm); or  _____ inch (_____ mm)>>​.

Adjusting

Adjust and align hardware to uniform clearance at vertical edge of doors, not exceeding << 3/16 inch (5 mm)>>.

Adjust hinges to position doors in ​<< partially open;  fully closed; or  _____>>​ position when unlatched. Return outswinging doors to closed position.

Adjust adjacent components for consistency of line or plane.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean compartment surfaces and hardware in accordance with manufacturer's instructions.

Protection

Protect installed compartments from subsequent construction operations.

- END OF SECTION